Control an Arduino Boost Converter from MATLAB

A beginner's step-by-step guide: set Kp, Ki and the target voltage from MATLAB, and see the results on your computer

# 1. What you will build

The Arduino runs a PI controller that keeps the converter's output voltage at a target value. With this guide, you will:

- Type the target voltage (Vref), Kp and Ki in MATLAB, with no need to change the Arduino code again.

- See the measured output voltage, duty cycle and PWM value in the Command Window and in a plot.

- Let MATLAB test many Kp and Ki values automatically and find the best pair.

# 2. Quick outline

- Gather the parts and connect the wires.

- Upload the Arduino code (only once).

- Run the MATLAB script and set Vref, Kp and Ki by hand.

- Read the results in the Command Window and the plot.

- (Optional) Run the auto-tuning script to find the best Kp and Ki.

- Fix problems with the troubleshooting table.

# 3. What you need

| Item | Notes |
| --- | --- |
| Arduino Uno | With a USB cable. |
| Boost converter power stage | Inductor, diode, output capacitor and a MOSFET switch module (for example LR7843) driven by the Arduino. |
| Voltage divider | R_TOP = 40 kOhm, R_BOT = 10 kOhm. The Arduino sees Vout / 5. |
| Potentiometer (optional) | Manual target knob on A4 (used only until MATLAB sends a target). |
| MATLAB | R2019b or newer (uses the serialport function). No extra toolbox needed. |
| Arduino IDE | To upload the code once. |

# 4. Connect the wires

| Arduino pin | Connect to |
| --- | --- |
| D9 | Signal input of the MOSFET module (PWM output). |
| A5 | Middle point of the voltage divider (between the 40k and 10k resistors). |
| A4 | Middle pin of the potentiometer (ends go to 5 V and GND). |
| GND | Ground of the converter and the divider's bottom resistor. All grounds must be common. |

Divider: converter output goes to the top of the 40k resistor, then 40k, then the A5 junction, then 10k, then GND. If you use other resistor values, change R_TOP and R_BOT in the Arduino code.

Important: use pin D9, not D5. D5 shares a timer with the Arduino clock functions and breaks the timing of the controller.

# 5. Upload the Arduino code (once)

- Open the Arduino IDE and paste the code below into a new sketch.

- Choose Tools > Board > Arduino Uno and select your COM port.

- Click Upload. Then close the Serial Monitor, because MATLAB needs the port.

#include <avr/io.h>

// ---------- Settings that MATLAB can change ----------

float KP = 0.1;

float KI = 40.0;

float remoteTargetV = 12.0;

bool  remoteMode = false;

const float MIN_VREF = 5.0, MAX_VREF = 18.0;

const float TS = 0.001;            // 1 ms control loop

const uint8_t MAX_DUTY = 200;      // limit (about 78 %)

// ---------- Pins ----------

const int FEEDBACK_PIN = A5;       // output voltage divider

const int POT_PIN      = A4;       // manual target knob

// PWM output is D9 (OC1A)

// Divider: R_TOP = 40k, R_BOT = 10k  -> ratio 5.0

const float R_TOP = 40.0, R_BOT = 10.0;

const float VOLTAGE_MULTIPLIER = (R_TOP + R_BOT) / R_BOT;

// ---------- Controller state ----------

float error = 0, prev_error = 0, control_y = 0, prev_y = 0;

float lastTargetV = 0;

int   lastAdc = 0;

char serialBuffer[40];

byte serialIndex = 0;

void handleCommand(char *cmd) {

if (strcmp(cmd, "RESET") == 0) {

control_y = 0; prev_y = 0; prev_error = 0;

OCR1A = 0;

Serial.println(F("ACK,RESET"));

return;

}

char *eq = strchr(cmd, '=');

if (eq == NULL) { Serial.println(F("ERROR,INVALID_COMMAND")); return; }

*eq = '\0';

float val = atof(eq + 1);

if (strcmp(cmd, "VREF") == 0) {

if (val >= MIN_VREF && val <= MAX_VREF) {

remoteTargetV = val; remoteMode = true;

Serial.print(F("ACK,VREF,")); Serial.println(val, 2);

} else Serial.println(F("ERROR,VREF_OUT_OF_RANGE"));

}

else if (strcmp(cmd, "KP") == 0) {

if (val >= 0 && val <= 5) {

KP = val;

Serial.print(F("ACK,KP,")); Serial.println(val, 4);

} else Serial.println(F("ERROR,KP_OUT_OF_RANGE"));

}

else if (strcmp(cmd, "KI") == 0) {

if (val >= 0 && val <= 500) {

KI = val;

Serial.print(F("ACK,KI,")); Serial.println(val, 4);

} else Serial.println(F("ERROR,KI_OUT_OF_RANGE"));

}

else Serial.println(F("ERROR,INVALID_COMMAND"));

}

void processSerialCommands() {

while (Serial.available() > 0) {

char c = Serial.read();

if (c == '\n' || c == '\r') {

if (serialIndex == 0) continue;

serialBuffer[serialIndex] = '\0';

handleCommand(serialBuffer);

serialIndex = 0;

} else if (serialIndex < sizeof(serialBuffer) - 1) {

serialBuffer[serialIndex++] = c;

} else {

serialIndex = 0;

}

}

}

void setup() {

Serial.begin(115200);

// Timer1 fast PWM on D9, about 62.5 kHz

DDRB |= (1 << PB1);

TCCR1A = (1 << COM1A1) | (1 << WGM10);

TCCR1B = (1 << WGM12)  | (1 << CS10);

OCR1A = 0;

Serial.println(F("READY"));

}

void loop() {

processSerialCommands();

// ---- Control loop: every 1 ms ----

static unsigned long lastCtl = 0;

if (micros() - lastCtl >= 1000) {

lastCtl += 1000;

float target_vout;

if (remoteMode) target_vout = remoteTargetV;

else target_vout = map(analogRead(POT_PIN), 0, 1023, 50, 180) / 10.0;

lastTargetV = target_vout;

float target_adc = (target_vout / VOLTAGE_MULTIPLIER / 5.0) * 1023.0;

lastAdc = analogRead(FEEDBACK_PIN);

error = target_adc - (float)lastAdc;

control_y = prev_y + KP * (error - prev_error) + KI * TS * error;

if (control_y > MAX_DUTY) control_y = MAX_DUTY;

else if (control_y < 0)   control_y = 0;

OCR1A = (uint8_t)control_y;

prev_error = error;

prev_y = control_y;

}

// ---- Telemetry: every 50 ms ----

static unsigned long lastPrint = 0;

if (millis() - lastPrint >= 50) {

lastPrint = millis();

float vout = (lastAdc / 1023.0) * 5.0 * VOLTAGE_MULTIPLIER;

float duty = ((float)OCR1A / 255.0) * 100.0;

Serial.print(F("DATA,"));

Serial.print(millis());       Serial.print(',');

Serial.print(lastTargetV, 2); Serial.print(',');

Serial.print(vout, 3);        Serial.print(',');

Serial.print(duty, 1);        Serial.print(',');

Serial.println(OCR1A);

}

}

## How the Arduino and MATLAB talk

| MATLAB sends | Meaning |
| --- | --- |
| VREF=12 | Set target voltage to 12 V (allowed 5 to 18). |
| KP=0.1 | Set proportional gain (allowed 0 to 5). |
| KI=20 | Set integral gain (allowed 0 to 500). |
| RESET | Set duty to 0 and clear the controller memory. |

The Arduino answers each command with ACK (accepted) or ERROR (rejected). It also sends a data line 20 times a second:

DATA,time_ms,target_V,Vout_V,duty_percent,PWM

# 6. Run the manual MATLAB script

This is the main script. You change the numbers at the top, press Run, and read the results.

- Open MATLAB and click New Script. Paste the code below and save it as boost_manual.m.

- Find your port: type serialportlist in the Command Window and put the result in the port line (for example "COM10").

- Change Vref, Kp, Ki and runTime in section 1.

- Click the green Run button (run the whole file, not just one section).

- Watch the table in the Command Window. At the end you get a summary and a plot.

clear; clc; close all;

%% ===== 1. SET YOUR VALUES HERE =====

Vref    = 12;      % target voltage (V), allowed 5 to 18

Kp      = 0.1;     % proportional gain, allowed 0 to 5

Ki      = 20;      % integral gain, allowed 0 to 500

runTime = 10;      % seconds to record

port    = "COM10"; % change to your Arduino port

%% ===== 2. CONNECT =====

s = serialport(port, 115200);

configureTerminator(s, "LF");

s.Timeout = 2;

pause(2.5);                  % wait for Arduino restart

flush(s);

%% ===== 3. SEND THE VALUES =====

sendCmd(s, "RESET");

sendCmd(s, sprintf("KP=%.4f", Kp));

sendCmd(s, sprintf("KI=%.4f", Ki));

sendCmd(s, sprintf("VREF=%.2f", Vref));

flush(s);

%% ===== 4. READ AND PRINT =====

fprintf("\n Time(s)  Target(V)  Vout(V)  Duty(%%)  PWM\n");

fprintf("------------------------------------------------\n");

data = [];

t0 = tic;

while toc(t0) < runTime

line = readline(s);

if isempty(line) || ismissing(line), continue; end

q = split(strtrim(line), ",");

if numel(q) ~= 6 || q(1) ~= "DATA", continue; end

v = str2double(q(2:6))';

if any(isnan(v)), continue; end

data = [data; toc(t0), v(2), v(3), v(4), v(5)]; %#ok<AGROW>

fprintf("%7.2f   %7.2f   %7.3f   %6.1f   %3d\n", data(end,:));

if v(3) > 20             % safety stop

sendCmd(s, "VREF=5"); sendCmd(s, "RESET");

warning("Vout above 20 V. Stopped for safety.");

break;

end

end

%% ===== 5. TURN OFF AND CLOSE =====

sendCmd(s, "VREF=5");

clear s;

%% ===== 6. SUMMARY =====

Time = data(:,1);  Target = data(:,2);  Vout = data(:,3);

Duty = data(:,4);  PWM = data(:,5);

finalVout = mean(Vout(max(1,end-10):end));

overshoot = max(0, (max(Vout) - Vref) / Vref * 100);

fprintf("\n===== SUMMARY =====\n");

fprintf("Kp = %.3f, Ki = %.1f, Vref = %.2f V\n", Kp, Ki, Vref);

fprintf("Final Vout  = %.3f V\n", finalVout);

fprintf("Final error = %.3f V\n", finalVout - Vref);

fprintf("Peak Vout   = %.3f V\n", max(Vout));

fprintf("Overshoot   = %.2f %%\n", overshoot);

%% ===== 7. PLOT =====

figure;

subplot(2,1,1);

plot(Time, Target, "--", Time, Vout, "LineWidth", 1.5); grid on;

legend("Target","Vout"); ylabel("Voltage (V)");

title(sprintf("Kp=%.3f  Ki=%.1f  Vref=%.1f V", Kp, Ki, Vref));

subplot(2,1,2);

plot(Time, Duty, "LineWidth", 1.5); grid on;

xlabel("Time (s)"); ylabel("Duty (%)");

%% ===== FUNCTION (keep at the bottom) =====

function ok = sendCmd(s, cmd)

ok = false;

flush(s);

writeline(s, cmd);

t0 = tic;

while toc(t0) < 1

line = readline(s);

if isempty(line) || ismissing(line), continue; end

line = strtrim(line);

if startsWith(line, "ACK"), ok = true; return; end

if startsWith(line, "ERROR"), warning("Arduino: " + line); return; end

end

warning("No reply to " + cmd);

end

## What you will see

- Command Window: a table of Time, Target, Vout, Duty and PWM, then a summary with final voltage, error and overshoot.

- Plot: target and output voltage on top, duty cycle below.

- Workspace: the variables Time, Target, Vout, Duty, PWM and data stay available for your own plots and calculations.

## Good first values

| Vref | Kp | Ki | Runtime |
| --- | --- | --- | --- |
| 8 to 12 V | 0.1 | 20 | 10 s |

Change one value at a time between runs, so you know which change caused a difference.

# 7. Optional: find the best Kp and Ki automatically

This script tests every Kp and Ki pair in your lists. Each test steps the target from Vlow to Vhigh and measures overshoot, steady-state error and total error. The pair with the lowest cost wins.

- Save the code below as boost_autotune.m and click Run.

- Start with the small lists shown (9 tests, about 1.5 minutes).

- To test more values, edit KpList and KiList. For a second, finer search, use numbers close to the best pair.

clear; clc; close all;

%% ===== SETTINGS =====

port   = "COM10";

Vlow   = 8;   Vhigh = 12;     % step from 8 V to 12 V

tLow   = 3;   tHigh = 5;      % seconds at each level

KpList = [0.05 0.1 0.2];      % Kp values to try

KiList = [10 20 40];          % Ki values to try

%% ===== CONNECT =====

s = serialport(port, 115200);

configureTerminator(s, "LF");

s.Timeout = 2;

pause(2.5);

flush(s);

%% ===== TEST EVERY Kp / Ki COMBINATION =====

results = [];

for kp = KpList

for ki = KiList

sendCmd(s, "RESET");

sendCmd(s, sprintf("KP=%.4f", kp));

sendCmd(s, sprintf("KI=%.4f", ki));

sendCmd(s, sprintf("VREF=%.2f", Vlow));

flush(s);

raw = []; sent = false; t0 = tic;

while toc(t0) < tLow + tHigh

if ~sent && toc(t0) >= tLow

writeline(s, sprintf("VREF=%.2f", Vhigh)); sent = true;

end

line = readline(s);

if isempty(line) || ismissing(line), continue; end

q = split(strtrim(line), ",");

if numel(q) ~= 6 || q(1) ~= "DATA", continue; end

v = str2double(q(2:6));

if any(isnan(v)), continue; end

raw = [raw; toc(t0), v(3)]; %#ok<AGROW>

if v(3) > 20

sendCmd(s, "VREF=5"); sendCmd(s, "RESET");

error("Vout above 20 V. Stopped for safety.");

end

end

k = raw(:,1) >= tLow;               % data after the step

t = raw(k,1) - tLow;  y = raw(k,2);

if numel(y) < 10, continue; end

overshoot = max(0, (max(y) - Vhigh) / (Vhigh - Vlow) * 100);

ssErr = abs(mean(y(round(0.8*end):end)) - Vhigh);

cost  = trapz(t, abs(y - Vhigh)) + 0.05*overshoot + 5*ssErr;

results = [results; kp, ki, overshoot, ssErr, cost]; %#ok<AGROW>

fprintf("Kp=%.3f Ki=%.1f  overshoot=%.1f%%  cost=%.3f\n", ...

kp, ki, overshoot, cost);

pause(1);

end

end

%% ===== SHOW THE BEST =====

results = array2table(results, "VariableNames", ...

["Kp","Ki","Overshoot","SteadyError","Cost"]);

results = sortrows(results, "Cost");

disp(results);

fprintf("BEST: Kp = %.3f, Ki = %.1f\n", results.Kp(1), results.Ki(1));

sendCmd(s, "VREF=5");

clear s;

%% ===== FUNCTION (keep at the bottom) =====

function ok = sendCmd(s, cmd)

ok = false;

flush(s);

writeline(s, cmd);

t0 = tic;

while toc(t0) < 1

line = readline(s);

if isempty(line) || ismissing(line), continue; end

line = strtrim(line);

if startsWith(line, "ACK"), ok = true; return; end

if startsWith(line, "ERROR"), warning("Arduino: " + line); return; end

end

warning("No reply to " + cmd);

end

## How to read the result

- Lower cost is better. Cost = total error + overshoot penalty + steady-state error penalty.

- A high Ki gives more overshoot. A very low Ki makes the response slow.

- If several pairs have almost the same cost, they are equally good. Pick the middle one.

- If the duty gets close to 78% at Vhigh, the converter cannot reach that voltage. Lower Vhigh.

# 8. Troubleshooting

| Problem | What to do |
| --- | --- |
| Port not found or busy | Close the Arduino Serial Monitor. Run clear all in MATLAB. Check the port with serialportlist. |
| Warning: No reply to ... | The Arduino has the old code, or the port or baud rate is wrong (use 115200 in both). |
| No data lines in the table | Wait for the pause(2.5) after connecting. Check the USB cable. |
| Vout stays near 0 V | Check the MOSFET wire is on D9, the converter has input power, and the A5 divider is wired. |
| Duty stuck at about 78% | The converter cannot reach the target with this input voltage or load. Lower the target. |
| Output is unstable | Lower Ki, then try a slightly higher Kp. |
| Error: Unrecognized function | Run the whole file with the green Run button. Do not name your file after a MATLAB function (for example report.m). |

# 9. Safety

- Start with low voltages and a small load.

- Never set a target above your capacitor and load ratings. The scripts stop if Vout goes above 20 V, but they cannot protect parts rated lower.

- Never leave a running converter unattended.

- A boost converter output is higher than its input. Keep hands away from live connections.

# 10. Quick cheat sheet

| I want to... | Do this |
| --- | --- |
| Change the target | Edit Vref in boost_manual.m and click Run. |
| Change Kp or Ki | Edit Kp or Ki at the top and click Run. |
| See numbers | Look at the Command Window table and summary. |
| Use the data | Use Time, Vout, Duty in the Workspace. |
| Find the best Kp and Ki | Run boost_autotune.m. |
| Change the Arduino limits | Edit the VREF, KP and KI range lines, then upload again. |